| - No. | ## Name | MATERIALS, dimension |
| --- | --- | --- |
| 1 | Keel | Oak laminate, 200x40 midships |
| 2 | Knee (bow transom) | pine, s=40 mm |
| 3 | Stern knee | pine, s=40 mm |
| 4 | Bulkhead (fr. А) | plywood, s=8mm |
| 5 | Frame А | pine 40 x 60 |
| 6 | Beam (fr. А) | pine 40 x 60 |
| 7 | Bulkhead (fr. B) | plywood, s=8mm |
| 8 | Frame B | pine 40 x 60 |
| 9 | Beam (fr. B) | pine 40 x 60 |
| 10 | Bunk framing (fr. B) | pine 20 x 30 |
| 11 | Bulkhead (fr. C) | plywood, s=8mm |
| 12 | Frame C | pine 40 x 60 |
| 13 | Floor (fr. C) | pine 40 x 80 |
| 14 | Bunk framing (fr. C) | pine 20 x 30 |
| 15 | Bulkhead (fr. D) | plywood, s=8mm |
| 16 | Frame D | pine 40 x 60 |
| 17 | Beam (fr. D) | pine 40 x 60 |
| 18 | Floor (fr. D) | pine 40 x 80 |
| 19 | Bunk framing (fr. D) | pine 20 x 30 |
| 20 | Knee (fr. D) | plywood, s=8mm |
| 21 | Bulkhead (fr. E) | plywood, s=8mm |
| 22 | Frame E | pine 40 x 60 |
| 23 | Floor (fr. E) | pine 40 x 80 |
| 24 | Bunk framing (fr. E) | pine 20 x 30 |
| 25 | Bulkhead (fr. F) | plywood, s=8mm |
| 26 | Frame F | pine 40 x 60 |
| 27 | Beam (fr. F) | pine 40 x 60 |
| 28 | Floor (fr. F) | pine 40 x 80 |
| 29 | Framing (fr. F) | pine 20 x 30 |
| 30 | Bulkhead (fr. G) | plywood, s=8mm |
| 31 | Frame G | pine 40 x 60 |
| 32 | Floor (fr. G) | pine 40 x 100 |
| 33 | Framing (fr. G) | pine 20 x 30 |
| 34 | Bulkhead (fr. H) | plywood, s=8mm |
| 35 | Frame H | pine 40 x 60 |
| 36 | Beam (fr. H) | pine 40 x 60 |
| 37 | Floor (fr. H) | pine 40 x 100 |
| 38 | Framing (fr. H) | pine 20 x 30 |
| 39 | Bulkhead (fr. I) | plywood, s=8mm |
| 40 | Frame I | pine 40 x 60 |
| 41 | Floor (fr. I) | pine 40 x 100 |
| 42 | Framing (fr. I) | pine 20 x 30 |
| 43 | Bulkhead (fr. J) | plywood, s=8mm |
| 44 | Frame (fr. J) | pine 40 x 60 |

| 45 | Beam (fr. J) | pine 40 x 60 |
| --- | --- | --- |
| 46 | Floor (fr. J) | pine 40 x 100 |
| 47 | Framing (fr. J) | pine 20 x 30 |
| 48 | Knee (fr. J) | plywood, s=8mm |
| 49 | Bulkhead (fr. K) | plywood, s=8mm |
| 50 | Frame (fr. K) | pine 40 x 60 |
| 51 | Floor (fr. K) | pine 40 x 100 |
| 52 | framing (fr. K) | pine 20 x 30 |
| 53 | Bulkhead (fr. L) | plywood, s=8mm |
| 54 | Frame (fr. L) | pine 40 x 60 |
| 55 | Beam (fr. L) | pine 40 x 60 |
| 56 | Floor (fr. L) | pine 40 x 100 |
| 57 | Framing (fr. L) | pine 20 x 30 |
| 58 | Knee (fr. L) | plywood, s=8mm |
| 59 | Hatch framing (fr. L) | pine 20 x 40 |
| 60 | Bulkhead (fr. M) | plywood, s=8mm |
| 61 | Frame (fr. M) | pine 40 x 60 |
| 62 | Beam (fr. M) | pine 40 x 60 |
| 63 | Floor (fr. M) | pine 40 x 100 |
| 64 | Framing (fr. M) | pine 20 x 30 |
| 65 | Knee (fr. M) | plywood, s=8mm |
| 66 | Bulkhead (fr. N) | plywood, s=8mm |
| 67 | Frame (fr. N) | pine 40 x 60 |
| 68 | Beam (fr. N) | pine 40 x 60 |
| 69 | Floor (fr. N) | pine 40 x 100 |
| 70 | Framing (fr. N) | pine 20 x 30 |
| 71 | Transom panel (fr. P) | plywood, s=8mm |
| 72 | Frame (fr. P) | pine 40 x 60 |
| 73 | Beam (fr. P) | pine 40 x 60 |
| 74 | Floor (fr. P) | pine 40 x 100 |
| 75 | Pillar (fr. P) | pine 40 x 150 |
| 80 | Sheer clamp | oak laminate 50 x 30 |
| 81 | Stringer | pine laminate 40 x 25 |
| 82 | Stringer | pine laminate 45 x 30 |
| 83 | Stringer | pine laminate 45 x 30 |
| 84 | Stringer | pine laminate 45 x 25 |
| 85 | Stringer | pine laminate 50 x 30 |
| 86 | Bunk overlay | plywood, s=8mm |
| 87 | Floorboard | plywood, s=8mm |
| 88 | Head side | plywood, s=8mm |
| 89 | Settee side | plywood, s=8mm |
| 90 | Settee overlay | plywood, s=8mm |
| 91 | Locker side | plywood, s=8mm |
| 92 | Pillar | steel d=50mm |
| 93 | Side galley | plywood, s=8mm |
| 94 | Settee side | plywood, s=8mm |
| 95 | Galley table | plywood, s=8mm |
| 96 | Settee overlay | plywood, s=8mm |
| 97 | Settee framing | pine 20 x 30 |
| 98 | Bunk overlay | plywood, s=8mm |
| 99 | Longitudinal bulkhead | plywood, s=8mm |
| 100 | Plank | pine 40 x 25 |
| 101 | Bunk carling | pine 20 x 30 |
| 102 | Afterpeak overlay | plywood, s=8mm |
| 103 | Plank | pine 50 x 25 |
| 104 | Plank | pine 40 x 25 |
| 105 | Longitudinal bulkhead | plywood, s=8mm |
| 106 | Bow transom piece | foam |
| 110 | Carling | pine 50 x 25 |
| 111 | Carling | pine 40 x 25 |
| 112 | Carling | pine 40 x 25 |
| 113 | Carling | pine 40 x 25 |
| 114 | Carling | pine 80 x 25 |
| 115 | Carling | pine 40 x 25 |
| 116 | Carling | pine 40 x 25 |
| 117 | Carling | pine 40 x 25 |
| 118 | Carling | pine 40 x 25 |
| 119 | Carling | pine 40 x 25 |
| 120 | Carling | pine 50 x 25 |
| 121 | Carling | pine 40 x 25 |
| 122 | Carling | pine 40 x 25 |
| 123 | Carling | pine 45 x 25 |
| 124 | Carling | pine 40 x 25 |
| 125 | Carling | pine 40 x 25 |
| 126 | Carling | pine 40 x 25 |
| 127 | Carling | pine 40 x 25 |
| 128 | Carling | pine 40 x 25 |
| 129 | Coaming framing | pine 20 x 20 |
| 130 | Stern compartment framing | pine 20 x 15 |
| 131 | Carling | pine 40 x 25 |
| 132 | Carling | pine 40 x 25 |
| 133 | Carling | pine 40 x 25 |
| 140 | Bottom plating | plywood, s=10mm |
| 141 | Chine 1 | plywood, s=8mm |
| 142 | Chine 2 | plywood, s=8mm |
| 143 | Board | plywood, s=8mm |
| 144 | Overlay deck | plywood, s=8mm |
| 145 | Superstructure coaming | plywood, s=8mm |
| 146 | Roof superstructure | plywood, s=8mm |
| 147 | ### Superstructure coaming | plywood, s=8mm |
| 148 | ### Superstructure coaming | plywood, s=8mm |
| 149 | ### Window | plexiglass, s=12mm |
| 150 | ### Cockpit plating | plywood, s=8mm |
| 151 | Cockpit side | plywood, s=8mm |
| 152 | ### Cockpit seat | plywood, s=8mm |
| 153 | Cockpit coaming | plywood, s=8mm |
| 154 | Cockpit coaming | plywood, s=8mm |
| 155 | Cockpit coaming | plywood, s=8mm |
| 156 | Cockpit coaming | plywood, s=8mm |
| 157 | Stern compartment cover | plywood, s=8mm |