BOAT: 			"PILGRIM 590"

DESIGNED BY: 	Albert Nazarov & Maxim Kovalyov (Sevastopol, UKRAINE)

nazarov@sevcable.net  www.nyd.da.ru

SPECIFICATIONS:

LAYOUT

1 – forepeak; 2 – berth; 3 – settee; 4 – galley; 5 – pillar; 6 – table; 7 – floorboard; 8 – centreboard trunk; 9 – centreboard; 10 – centreboard control lines; 11 – step; 12 – outboard motor stowage; 13 – outboard ladder; 14 – cockpit; 15 – mast step; 16 – cabin hatch; 17 – steering; 18 – bow pulpit; 19 – pulpit; 20 – winch; 21 – cleat; 22 – toe rail; 23 – mooring cleat; 24 – mainsheet rail; 25 – jib sheet block; 26 – outboard motor bracket; 27 – chainplate;  28 – flotation element; 29 – inner ballast; 30 – shelf; 31 – stove; 32 – locker

CONSTRUCTION

1- bow transom (A), plywood, s=10mm; 2- beam, pine, 30х60mm; 3-frame, pine, 30х60 mm; 4- stringer, pine laminate, 25х40mm; 5-sheer clamp, oak, 25х60mm; 6- forefoot, oak laminate, s=40 mm; 7- bow transom knee, oak, s=30 mm; 8- bulkhead (B), plywood, s=10mm; 9- plank, pine, 20х30mm; 10- berth plating, plywood, s=8mm; 11- floortimber, pine, 30х100mm; 12- knee (both sides), plywood, s=6mm; 13- keel, oak laminate, s=30mm; 14- carling, pine, 25х40mm; 15- bulkhead (C), plywood, s=10mm; 16- galley side, plywood, s=6mm; 17- galley table, plywood, s=6mm; 18- plank, pine, 25х40mm; 19- centreboard trunk side, plywood, s=8mm; 20- carling, pine 25х80mm; 21- floor, plywood, s=10mm; 22- floor, plywood, s=10mm; 23- floor, plywood, s=10mm; 24- floorboard, plywood, s=10mm; 25- bulkhead (D), plywood, s=10mm; 26- pillar, steel tube d=32mm; 27- pillar, pine, 30х100; 28- bulkhead (E), plywood, s=10mm; 29- hatch framing, 30х40mm; 30- halfbeam, pine,30х60mm; 31- bulkhead (F), plywood, s=10mm; 32- pillar, pine, 30х60 mm; 33- bulkhead (G), plywood, s=8mm; 34- berth plating, plywood, s=8mm; 35- beam, s=30mm; 36- pillar, pine, 30х150mm; 37- stern knee, oak, 30х120х120mm; 38- transom plating, plywood, s=8mm; 39- transom (H), plywood, s=10mm; 40- skeg, oak, s=40 mm;41- bulkhead of outboard motor compartment, plywood, s=8mm; 42- longitudinal bulkhead, plywood, s=8mm; 43- bow piece, foam; 44- bottom plating, plywood, s=8mm; 45- bottom plating, plywood, s=8mm; 46- chine plating, plywood, s=6mm; 47- board plating, plywood, s=6mm; 48- deck plating, plywood, s=8mm; 49- deck chamfer, plywood, s=6mm; 50- roof superstructure, plywood, s=10mm; 51- superstructure coaming, plywood, s=6mm; 52- superstructure coaming, plywood, s=6mm; 53- cockpit seat, plywood, s=8mm; 54- cockpit side, plywood, s=6mm; 55- cockpit plating, plywood, s=8mm; 56- cockpit coaming, plywood, s=4mm; 57- window, plexiglass, s=10mm 58- centerboard, alloy or steel, s=10 mm.

MATERIALS*

| Name | Size | Quantity | Notes |
| --- | --- | --- | --- |
| Plywood | 1.25x2.50m | 26 sheets |  |
| - 10 mm |  |  |  |
| - 8 mm |  |  |  |
| - 6 mm |  |  |  |
| Timber: |  |  |  |
| - pine |  | 0.5 m3 |  |
| - oak |  | 0.11 m3 |  |
| Epoxy resin |  | 150 kg |  |
| Glass cloth | 100…200 g/m2 | 150 m2 |  |

* in-process losses considered

CORRECTIONS

| Item to be corrected | Currently printed | Should be printed | Notes |
| --- | --- | --- | --- |
| Frame E, overall vertical dimension | 1752 | 1652 |  |

Please, inform us about boats in construction. Thanks for corrections if any.